ECO-FAST

Economically and Environmentally Conscious
Food Waste Assessment and Selection Tool

TOOL DESCRIPTION REPORT

Version 1.0   —   July 2026

# Table of Contents

1.  Overview and Purpose

2.  System Architecture

3.  Technology Superstructure

4.  Tab 1 — Instructions

5.  Tab 2 — Feed Inputs

6.  Tab 3 — Technology Specifications

7.  Tab 4 — Cost Specifications

8.  Tab 5 — Results

9.  Tab 6 — Environmental Justice Assessment

10.  Mathematical Model Summary

11.  Data Flow Between Tabs

12.  Software Stack and Deployment

# 1.  Overview and Purpose

ECO-FAST (Economically and Environmentally Conscious Food Waste Assessment and Selection Tool) is a web-based decision-support platform developed to assist engineers, municipalities, waste management authorities, and policymakers in identifying the most economically viable and environmentally responsible approach for processing a given food waste stream. The tool is designed to be accessible to users without deep optimisation expertise, while providing rigorous, mathematically grounded results suitable for academic publication and regulatory decision-making.

Given a characterised food waste stream — defined by its flow rate, wet-basis composition, and facility location — ECO-FAST evaluates all feasible combinations of pretreatment, conversion, and product recovery technologies simultaneously using a Mixed-Integer Nonlinear Programming (MINLP) optimisation model. The model selects the optimal technology pathway and computes the associated annualised costs and greenhouse gas (GHG) emissions for three objective modes:

Lowest Cost Pathway:  Identifies the processing configuration that minimises Net Annual Cost (NAC, M$ yr⁻¹) without any constraint on GHG emissions. This represents the economically dominant solution.

Lowest Emissions Pathway:  Identifies the configuration that minimises total life-cycle GHG emissions (t CO₂-eq yr⁻¹) without constraint on cost. This represents the environmentally dominant solution.

Lowest Cost and Emissions Pathway:  Applies the epsilon-constraint multi-objective method to generate the full Pareto frontier of cost-versus-emissions trade-offs. Each point on the frontier is Pareto-optimal — no other feasible configuration can improve one objective without worsening the other. This mode supports decision-making where both economic and environmental performance must be balanced.

The tool is intended for use at the facility-planning stage, where a waste management authority or developer is evaluating which processing technology to invest in. Results are expressed in annualised terms at the design throughput. Sensitivity to economic assumptions (prices, discount rate, plant lifetime) can be explored by adjusting parameters in Tabs 3 and 4 and re-running the optimisation.

# 2.  System Architecture

ECO-FAST follows a two-process client-server architecture. Both processes must run simultaneously on the same machine for the tool to function correctly.

## 2.1  Frontend — Streamlit (Port 8501)

The user-facing web application is built with Streamlit and served at http://localhost:8501. It renders all six tabs, collects user inputs through interactive widgets, sends optimisation requests to the backend as JSON payloads via the FastAPI REST API, and displays results including tables, charts, and interactive maps. All inputs entered during a session are held in Streamlit session state and are lost when the browser is closed or the page is refreshed.

## 2.2  Backend — FastAPI (Port 8000)

A REST API server built with FastAPI and served by Uvicorn at http://127.0.0.1:8000. It receives optimisation requests from the frontend, builds the full GAMSPy MINLP model from the provided inputs, executes the solver, and returns results as a structured JSON response. The backend is stateless; each request builds and solves a fresh model instance.

## 2.3  Optimisation Model Package (model/)

The mathematical model is implemented as a Python package under the model/ directory comprising seven modules:

sets.py:  Defines all index sets (technologies, stream components, product types).

parameters.py:  Declares all scalar and indexed parameters with literature-based default values.

variables.py:  Declares all decision variables (binary technology selectors, continuous mass flows, economic and emission quantities).

equations.py:  Implements all mass balance, process constraint, and sizing equations across all 15 technology units.

costing.py:  Computes all eight cost components (capital, working capital, insurance, utility, labour, overhead, raw material, disposal) and total revenue.

ghg.py:  Computes all GHG emission terms (indirect, direct, aqueous-phase, displacement credits).

solver.py:  Implements the three objective modes, the epsilon-constraint Pareto logic, and the run_optimization() entry point called by the API.

# 3.  Technology Superstructure

The superstructure represents all possible processing pathways considered by the optimisation model. It is organised into four sequential stages connected by 61 numbered material streams. Each stream carries component-resolved mass flow rates (kg hr⁻¹) across eight components: water (WATER), carbohydrates (CBH), protein (PRT), fat/lipid (FAT), other organics (OTH), ash (ASH), fixed carbon (FC), and enzyme (ENZYME). Binary decision variables select one configuration at each stage, enforcing mutual exclusivity through equality constraints.

## 3.1  Stage 1 — Mechanical Pretreatment

One of three mechanical pretreatment options is selected by binary variable y_{s1,m} in {0,1}:

Shredding (SHR):  Dry particle-size reduction. Specific power 0.05 kW per kg hr⁻¹ throughput. Suitable for all downstream conversion pathways.

Maceration (MCR):  Wet grinding with water addition (default ratio 2.0 kg water per kg dry feed) producing a pumpable slurry. Used primarily ahead of HTL and AND.

Bypass (BYP):  Feed passes directly to biological pretreatment or conversion without mechanical treatment. Required for landfill and incineration pathways.

## 3.2  Stage 2 — Biological Pretreatment

An optional biological pretreatment step conditions the feed prior to conversion. One of three options is selected:

Aerobic Biodigestion (AER):  Partial aerobic degradation of volatile solids fractions at component-specific degradation efficiencies (CBH 0.40, PRT 0.30, FAT 0.15, OTH 0.20 by default). Produces a CO₂/H₂O offgas and a conditioned slurry fed to conversion.

Enzymatic Hydrolysis (ENZ):  Cellulase enzyme addition (default dose 0.02 kg enzyme per kg dry solids) that hydrolyses carbohydrates into fermentable sugars, improving biogas yields in downstream AND. Hydrolysis efficiency default 0.85.

Bypass (BYP_bio):  Feed passes directly to the conversion stage. Required for CMP, SLF, INC, and WWT pathways.

## 3.3  Stage 3 — Conversion

The primary decision of the model: one of six mutually exclusive conversion technologies processes the conditioned feed.

| Technology | Abbreviation | Primary Product | Key Parameters |
| --- | --- | --- | --- |
| Hydrothermal Liquefaction | HTL | Biocrude oil | Temperature 300-350 C; pressure 150-250 bar; biocrude yield 0.35 (default) |
| Anaerobic Digestion | AND | Biomethane | BMP 400 mL CH4/g VS; VS degradation efficiency 0.85; HRT 720 hr |
| Composting | CMP | Compost | VS degradation 50%; composting time 120 hr; IPCC EFs for CH4 and N2O |
| Sanitary Landfill | SLF | Landfill gas (CH4) | IPCC first-order decay; gas capture efficiency 0.65; MCF 0.60 |
| Wastewater Treatment | WWT | Biosolids | F/M ratio 0.30 kg BOD/kg MLSS/day; MLSS 3,000 mg/L; SRT 10 days |
| Incineration | INC | Electricity (via STB) | Excess air ratio lambda=1.2; elemental combustion stoichiometry; coupled to steam turbine |

## 3.4  Stage 4 — Product Recovery and Upgrading

Downstream units recover and upgrade conversion products to marketable form:

Centrifugation (CEN) / Filtration (FLT):  Separate biocrude, char, and aqueous phases from the HTL product stream. CEN achieves 95% biocrude recovery; FLT achieves 98%. One unit is selected when HTL is the chosen conversion pathway.

Amine Scrubbing (ABS):  Chemical absorption of CO₂ from biogas to produce pipeline-quality biomethane (>=96% CH₄ purity). CH₄ slip 0.1%, CO₂ removal 98.5%, electricity use 0.12 kWh m⁻³.

Pressure Swing Adsorption (PSA):  Physical adsorption of CO₂ from biogas. CH₄ slip 1.5%, CO₂ removal 97%, electricity use 0.25 kWh m⁻³.

Steam Turbine (STB):  Recovers energy from INC flue gas to raise superheated steam (h1 = 3,277.9 kJ/kg) that expands through a back-pressure turbine (eta_turbine = 0.85, eta_generator = 0.95, h2 = 2,800 kJ/kg) to generate electricity.

# 4.  Tab 1 — Instructions

The Instructions tab is the landing page of ECO-FAST. It requires no user input and is intended to orient new users before they begin entering data. The tab contains the following content:

Tool purpose statement:  A concise description of what ECO-FAST does and the three objective modes available (Lowest Cost, Lowest Emissions, Lowest Cost and Emissions).

Superstructure diagram:  A graphical representation of the full technology network (loaded from Superstructure.tif). The diagram shows all 15 processing units and their interconnections across all four processing stages.

Technology summary panels:  Three side-by-side panels listing the technologies available at each stage: Pretreatment (SHR, MCR, AER, ENZ), Conversion (HTL, AND, CMP, SLF, WWT, INC), and Recovery and Upgrading (CEN, FLT, ABS, PSA, STB).

Step-by-step usage guide:  Five numbered instructions directing the user through Tabs 2 to 6 in sequence.

# 5.  Tab 2 — Feed Inputs

The Feed Inputs tab is the primary data entry interface where the user characterises the food waste stream entering the system. All values entered here are stored in session state and passed to the optimisation backend at the time of execution. The tab is divided into a left input panel and a right visualisation panel.

## 5.1  Food Waste Type

The user selects a food waste category from a predefined library. The selection automatically populates all composition fields with representative wet-basis mass fractions from the published literature. A 'Custom (Enter Your Own Data)' option unlocks all seven composition fields for manual entry. The tool does not allow the user to proceed further until a waste type is selected.

## 5.2  Feed Stream Conditions

| Input | Unit Options | Default | Description |
| --- | --- | --- | --- |
| Feed flow rate | kg/hr, t/day, lb/hr, kg/day | 1,000 kg/hr | Mass flow rate of the wet food waste feed stream entering the system. Converted internally to kg/hr for all calculations. |
| Annual operating hours | hr/yr, days/yr, weeks/yr | 7,920 hr/yr | Total hours per year the facility operates. Default corresponds to 330 days/yr of continuous operation. Converted internally to hr/yr. |

## 5.3  Facility Location

The user enters a five-digit US zip code identifying the proposed facility location. This field is optional for the optimisation but required for the Environmental Justice assessment in Tab 6. The zip code is used to retrieve geographic coordinates from the Zippopotam.us API and demographic data from the US Census Bureau ACS API. If no zip code is entered, Tab 6 will remain inactive.

## 5.4  Wet-Basis Composition

Seven component mass fractions are entered on a wet basis. When a predefined waste type is selected, these fields are auto-populated and locked. For the custom option, all fields are editable. A running total is displayed; the tool raises an error and blocks execution if the sum deviates from 1.0 by more than 0.01.

| Component | Symbol | Typical Range (wet basis) | Role in the Model |
| --- | --- | --- | --- |
| Water | WATER | 0.60 - 0.85 | Determines drying and evaporation loads. Dilutes organic content, reducing volumetric energy density and biogas yield per unit feed. |
| Carbohydrates | CBH | 0.01 - 0.12 | Primary substrate for biological conversion pathways. Governs BMP contribution from carbohydrates and AER/ENZ degradation kinetics. |
| Protein | PRT | 0.02 - 0.15 | Key nitrogen source. Determines C/N ratio (important for composting stability and AD inhibition risk). Participates in AER and ENZ degradation. |
| Fat / Lipid | FAT | 0.01 - 0.08 | Highest energy density of all components. Disproportionately contributes to biocrude yield in HTL and to methane yield in AND due to high BMP of lipids. |
| Other Organics | OTH | 0.01 - 0.05 | Catch-all for non-classified organic matter (e.g. cellulose, lignin, mixed organics). Participates in all biological and thermal conversion pathways. |
| Ash | ASH | 0.005 - 0.02 | Inert mineral content. Passes through all conversion units unchanged and reports to residue, char, or biosolids output streams. |
| Fixed Carbon | FC | 0.01 - 0.05 | Stable carbonaceous material (e.g. charcoal-like fractions). Slow to degrade biologically; contributes to char yield in HTL and increases ash-like residue in INC. |

## 5.5  Feedstock Quality Indicators

The right panel automatically computes and displays four quality indicators from the entered composition. These metrics give the user an immediate sense of which processing pathways are likely to be feasible and economically attractive for their specific feedstock.

| Indicator | Basis | Interpretation |
| --- | --- | --- |
| Higher Heating Value (HHV, MJ/kg) | Correlation based on organic component fractions | Gross energy content of the feed. Higher HHV favours INC and HTL pathways for energy recovery. |
| Volatile Solids (VS, %) | (CBH + PRT + FAT + OTH) x 100 | Biodegradable organic fraction. Primary basis for BMP and degradation calculations in AND, AER, CMP, and WWT. |
| Total Solids (TS, %) | (1 - WATER) x 100 | Dry matter content. Low TS (high moisture) limits thermochemical conversion efficiency but suits aqueous processes like HTL and WWT. |
| C/N Ratio | Carbon in organics / Nitrogen in protein | Indicator of composting suitability (optimal 25-35:1) and AD stability. High C/N may inhibit microbial activity in biological pathways. |

A donut chart in the right panel visualises the wet-basis composition breakdown by component with a colour-coded legend, updating in real time as the user changes composition values.

# 6.  Tab 3 — Technology Specifications

The Technology Specifications tab provides access to the engineering parameters for each of the 15 processing units in the superstructure. Users can review and adjust default values before running the optimisation. Default values are drawn from peer-reviewed literature, IPCC 2006 Tier 1 guidelines, and standard chemical engineering references. A dropdown menu at the top of the tab selects the technology; the corresponding parameter panel appears below.

## 6.1  Hydrothermal Liquefaction (HTL)

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| Biocrude yield fraction | - | 0.35 | Mass fraction of DAF feed converted to biocrude oil product |
| Char yield fraction | - | 0.10 | Mass fraction converted to solid char by-product |
| Aqueous phase yield fraction | - | 0.40 | Mass fraction reporting to the aqueous by-product phase |
| Gas product yield fraction | - | 0.15 | Mass fraction released as non-condensable gas; sum of four yield fractions must equal 1.0 |
| Reaction temperature | C | 340 | HTL reactor operating temperature; 280-370 C range for food waste |
| Residence time | min | 60 | Mean residence time in the HTL reactor |
| Dry solid loading | % | 7.0 | Mass fraction of dry solids in the feed slurry entering the reactor |

## 6.2  Anaerobic Digestion (AND)

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| Biochemical methane potential (BMP) | mL CH4/g VS | 400 | Maximum specific methane yield under batch test conditions; represents the biodegradability of the VS fraction. Literature range for food waste: 357-490 mL CH4/g VS. |
| VS degradation efficiency | - | 0.85 | Fraction of BMP achieved in a continuous digester; accounts for kinetic limitations relative to the batch BMP assay |
| Biogas capture efficiency | - | 0.98 | Fraction of produced biogas successfully collected |
| Other organics degradation fraction | - | 0.30 | Fraction of OTH component degraded in the digester |
| Hydraulic retention time (HRT) | hr | 720 | Mean residence time of feed in the digester (30 days default) |
| Organic loading rate (OLR) | kg VS/m3/day | 2.5 | Mass of volatile solids fed per unit reactor volume per day; determines reactor volume |
| Vessel fill fraction | - | 0.85 | Fraction of the reactor volume available for liquid; accounts for headspace |

## 6.3  Aerobic Biodigestion (AER)

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| Carbohydrate degradation fraction (fdeg_CBH) | - | 0.40 | Fraction of carbohydrates aerobically degraded to CO2 and H2O |
| Protein degradation fraction (fdeg_PRT) | - | 0.30 | Fraction of protein degraded aerobically |
| Fat degradation fraction (fdeg_FAT) | - | 0.15 | Fraction of fat degraded aerobically |
| Other organics degradation (fdeg_OTH) | - | 0.20 | Fraction of other organics degraded aerobically |
| Hydraulic retention time | hr | 24 | Residence time in the aerobic biodigester |
| Water addition ratio | kg/kg dry feed | 1.5 | Water added to maintain slurry consistency for pumping |

## 6.4  Enzymatic Hydrolysis (ENZ)

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| Hydrolysis efficiency (eta_ENZ) | - | 0.85 | Fraction of carbohydrates successfully hydrolysed to soluble sugars |
| Enzyme dose (r_enz) | kg enzyme/kg dry solids | 0.02 | Mass of cellulase enzyme added per kg of dry feed solids |
| Hydraulic retention time (HRT_ENZ) | hr | 6.0 | Contact time between enzyme and substrate |

## 6.5  Composting (CMP)

| Parameter | Unit | Default | Source |
| --- | --- | --- | --- |
| Volatile solids degradation fraction | - | 0.50 | Standard composting literature |
| Other organics degradation fraction | - | 0.60 | Process-specific estimate |
| Composting time | hr | 120 | Typical active composting phase duration (5 days) |
| CH4 emission factor | kg CH4/kg wet feed | 0.004 | IPCC 2006 Tier 1 default for aerobic composting |
| N2O emission factor | kg N2O/kg wet feed | 0.0003 | IPCC 2006 Tier 1 default for aerobic composting |

## 6.6  Wastewater Treatment (WWT)

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| BOD fraction of volatile solids (fBOD) | - | 1.20 | Ratio of biochemical oxygen demand to volatile solids mass; reflects oxygen demand of the organic substrate |
| Sludge retention time (SRT) | days | 10 | Mean residence time of biological solids in the activated sludge system; governs biosolids production via yield/decay model |
| MLSS concentration (X_MLSS) | g/L | 3.0 | Mixed liquor suspended solids target concentration; typical activated sludge range 2,000-4,000 mg/L |
| Minimum HRT | hr | 6.0 | Minimum hydraulic retention time constraint in the aeration basin |

## 6.7  Sanitary Landfill (SLF)

| Parameter | Unit | Default | Source |
| --- | --- | --- | --- |
| Degradable organic carbon fraction (DOC) | - | 0.358 | IPCC 2006 Tier 1 default for food waste |
| Fraction of DOC that decomposes (DOC_f) | - | 0.77 | IPCC 2006 Tier 1 default |
| Methane correction factor (MCF) | - | 0.60 | IPCC 2006 Tier 1 default for managed anaerobic landfill |
| CH4 fraction in landfill gas (F) | - | 0.576 | IPCC 2006 Tier 1 default |
| Gas capture efficiency | - | 0.65 | Typical value for landfills with active gas collection |
| Oxidation factor (OX) | - | 0.10 | IPCC 2006 Tier 1 default; fraction of CH4 oxidised in landfill cover |
| Landfill depth | m | 10 | Used in area-based sizing calculations |

## 6.8  Incineration (INC)

INC uses a first-principles elemental combustion model rather than a fixed conversion efficiency. Flue gas composition is computed from mole balances on C, H, O, N, and S using the dry-basis ultimate analysis of the feed. The three adjustable parameters are:

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| Combustion efficiency | - | 0.98 | Fraction of combustible material fully oxidised in the furnace |
| Boiler efficiency | - | 0.80 | Fraction of flue gas thermal energy transferred to the steam circuit |
| Excess air ratio (lambda) | - | 1.20 | Ratio of actual air supplied to stoichiometric air requirement; 1.20 means 20% excess air |

## 6.9  Gas Upgrading (ABS and PSA)

| Parameter | ABS Default | PSA Default | Description |
| --- | --- | --- | --- |
| CO2 removal efficiency | 98.5% | 97.0% | Fraction of CO2 removed from the raw biogas stream |
| CH4 slip fraction | 0.1% | 1.5% | Fraction of CH4 lost to the off-gas (not recovered in product stream) |
| Electricity consumption | 0.12 kWh/m3 | 0.25 kWh/m3 | Specific electricity use per m3 of raw biogas processed at standard conditions |
| Required biomethane purity | 96% CH4 | 96% CH4 | Minimum molar fraction of CH4 in the product stream (enforced as a model constraint) |

## 6.10  Steam Turbine (STB)

| Parameter | Unit | Default | Description |
| --- | --- | --- | --- |
| Turbine isentropic efficiency (eta_t) | - | 0.85 | Ratio of actual work extracted to isentropic (ideal) work; represents mechanical losses in the turbine |
| Generator efficiency (eta_g) | - | 0.95 | Fraction of shaft power converted to electrical power by the generator |
| Turbine inlet enthalpy (h1) | kJ/kg | 3,277.9 | Specific enthalpy of superheated steam entering the turbine; corresponds to approximately 60 bar and 450 C |
| Turbine outlet enthalpy (h2) | kJ/kg | 2,800 | Isentropic outlet enthalpy; corresponds to back-pressure exhaust at approximately 9-10 bar |
| Flue gas inlet temperature | C | 850 | Temperature of INC flue gas entering the waste heat recovery boiler |
| Flue gas outlet temperature | C | 400 | Temperature of cooled flue gas leaving the boiler |
| Flue gas heat capacity (Cp) | kJ/kg C | 1.15 | Specific heat of combustion flue gas at elevated temperature |

# 7.  Tab 4 — Cost Specifications

The Cost Specifications tab collects the economic parameters and solver settings required to run the optimisation. It is organised into three sections: economic parameters, product prices and revenue preview, and solver configuration.

## 7.1  Economic Parameters

| Parameter | Unit | Default | Role in Model |
| --- | --- | --- | --- |
| Electricity cost (C_elec) | $/kWh | 0.10 | Unit purchase price of grid electricity. Applied to net electricity consumption (PW_total) in the utility cost equation: CC_UC = C_elec x PW_net x T_ann. |
| Tipping fee (C_tip) | $/kg feed | 0.08 | Revenue received per kg of food waste accepted at the gate. Included in the REV calculation: REV_tip = C_tip x Q_f x T_ann / 10^6. |
| Labor cost (C_lbr) | $/hr | 30 | Hourly wage rate for facility operators. Labour cost is scaled by the number of workers (N_lbr) determined from facility size correlations. |
| Discount rate (r) | % yr-1 | User defined | Annual discount rate used to compute the Capital Recovery Factor (CRF = r(1+r)^n / ((1+r)^n - 1)). |
| Plant lifetime (n) | yr | User defined | Economic project lifetime. Together with r, determines CRF and hence the annualised capital cost (CC_AC). |

## 7.2  Product Prices and Revenue Preview

The user sets the market selling price for each saleable product. A live revenue calculator below the input fields computes and displays the estimated annual revenue from each product at the current feed rate and composition (using pre-set yield assumptions), allowing the user to assess the economic significance of each revenue stream before committing to a full optimisation run.

| Product | Unit | Typical Range | Notes |
| --- | --- | --- | --- |
| Biocrude | $/kg | 0.30 - 0.60 | Crude bio-oil from HTL pathway. Displaces fossil crude in the GHG displacement credit calculation. |
| Biomethane | $/kg | 0.50 - 1.20 | Upgraded biogas (>=96% CH4) from AND or SLF pathways. Displaces fossil natural gas. |
| Compost | $/kg | 0.03 - 0.15 | Stabilised organic matter from the CMP pathway. Displaces synthetic fertiliser in GHG accounting. |
| Biosolids | $/kg | 0.01 - 0.10 | Dewatered sludge from WWT (and HTL aqueous treatment). Displaces mineral soil amendments. |
| Electricity | $/kWh | 0.08 - 0.15 | Generated by the steam turbine in the INC pathway. Displaces grid electricity. |

## 7.3  Solver Configuration

| Setting | Options / Range | Default | Effect on Computation |
| --- | --- | --- | --- |
| Optimisation objective | Lowest Cost / Lowest Emissions / Balanced | Balanced | Selects which of the three solver modes is executed. 'Lowest Cost' and 'Lowest Emissions' each perform one MINLP solve. 'Balanced' performs N + 2 solves (2 anchors + N Pareto points). |
| Number of Pareto points (N) | 2 - 20 | 10 | Number of epsilon-constraint grid points used to trace the Pareto frontier. More points give a smoother and more informative frontier but increase total solve time approximately linearly. |
| Time limit per solve | 30 - 600 seconds | 300 s | Maximum CPU time allocated to each individual MINLP solve. Tighter limits reduce computation time but may yield solutions with larger optimality gaps. |
| Relative optimality gap | 0.01 - 0.20 | 5% (0.05) | The solver terminates when the best feasible solution found is within this percentage of the best theoretical lower bound. A 5% gap is standard for engineering optimisation problems. |

# 8.  Tab 5 — Results

The Results tab is the core output interface of ECO-FAST. The user clicks 'Run Optimisation' to send all parameters from Tabs 2-4 to the FastAPI backend as a JSON payload. The backend assembles and solves the MINLP model and returns the Pareto results. Results are displayed in several sections within the same tab.

## 8.1  Pathway Configuration

For each Pareto point, the complete technology configuration is reported: selected conversion technology, biological pretreatment unit, mechanical pretreatment unit, HTL recovery unit (if applicable), gas upgrading unit (if applicable), and steam turbine status (INC pathway only). The associated NAC (M$ yr-1) and GHG (t CO2-eq yr-1) values are shown for each configuration.

## 8.2  Cost Breakdown

The total annual cost (CCTC) is decomposed into eight components for the selected pathway:

| Symbol | Cost Component | Basis of Calculation |
| --- | --- | --- |
| CC_AC | Annualised Capital Cost | Equipment purchase cost scaled by the six-tenths law (exponent 0.67), multiplied by BMC = 5.4 (bare module multiplier), 1.66 (contingency/fees), and CRF. |
| CC_WC | Working Capital Cost | Fraction of total capital held as liquid operating reserve (typically 10-15% of CCTC). |
| CC_INS | Insurance Cost | Annual insurance premium, typically 1-2% of total installed capital. |
| CC_UC | Utility Cost | Cost of net electricity purchased from external sources: C_elec x PW_net x T_ann. |
| CC_LB | Labour Cost | Direct operator labour: N_workers x C_lbr x T_ann, where N_workers is scaled by facility throughput. |
| CC_OC | Overhead Cost | Indirect operating costs including maintenance, administration, and supervision (typically 60-80% of labour cost). |
| CC_RM | Raw Material Cost | Cost of consumables such as enzymes (ENZ pathway), chemicals, and media. |
| CC_DISP | Waste Disposal Cost | Cost for disposal of residue streams including non-captured char, ash, and process effluents. |

## 8.3  GHG Breakdown

Total GHG emissions (t CO2-eq yr-1) are decomposed into four terms:

| Symbol | GHG Component | Description |
| --- | --- | --- |
| GHG_ind | Indirect Emissions | Scope 2 emissions from grid electricity consumed by all processing units, multiplied by the grid emission factor (phi_elec, kg CO2-eq/kWh). |
| GHG_dir | Direct Process Emissions | Scope 1 emissions including process CO2 from biological degradation, fugitive CH4 (GWP = 28) and N2O (GWP = 265, IPCC AR5) from biological and thermal units, and flue gas CO2 from incineration. |
| GHG_aq | Aqueous-Phase Emissions | Emissions associated with treatment and disposal of aqueous by-product streams (primarily from HTL and WWT). |
| GHG_disp | Displacement Credits (subtracted) | Avoided fossil emissions credited for replacing fossil products: biocrude displaces crude oil, biomethane displaces natural gas, electricity displaces grid power, compost displaces synthetic fertiliser, biosolids displace mineral amendments. |

## 8.4  Pareto Frontier Plot

For the Balanced objective mode, a cost-versus-emissions scatter plot displays all N Pareto-optimal solutions, coloured by conversion technology. The shape of the frontier reveals the magnitude of the cost penalty associated with reducing GHG emissions and identifies 'knee points' where large emission reductions can be achieved at relatively modest additional cost.

## 8.5  Results Download

All Pareto results - including pathway configuration, all eight cost components, all four GHG components, NAC, REV, and CCTC per Pareto point - are available for download as pareto_results.csv for further analysis in Excel, Python, or R.

# 9.  Tab 6 — Environmental Justice Assessment

The Environmental Justice (EJ) tab assesses the equity implications of siting a food waste facility at the specified location by combining optimisation results with community demographic vulnerability data. The tab is active only when a valid zip code has been entered in Tab 2 and an optimisation has been completed in Tab 5. The methodology follows the EPA EJScreen framework (US EPA, 2023).

## 9.1  Facility Location and Buffer Zones

An interactive map rendered with Folium displays the facility location with three concentric buffer zones:

| Buffer Zone | Radius | Colour | Interpretation |
| --- | --- | --- | --- |
| Immediate impact zone | 1 km | Red | Residents within this radius experience the most direct exposure to facility emissions, traffic, and noise. |
| Surrounding neighbourhood | 3 km | Orange | The area most commonly affected by atmospheric dispersion of air pollutants under typical meteorological conditions. |
| Broader community | 5 km | Green | The wider area potentially affected by cumulative environmental burdens from facility operations. |

Note: the buffer rings are illustrative. Demographic statistics are reported at the zip code tabulation area (ZCTA) level from the US Census ACS 5-year estimates - not computed separately for each buffer radius.

## 9.2  Community Demographics

Demographic data for the facility zip code are retrieved from the US Census Bureau ACS 5-year estimates (2023 vintage, accessed via the Census Data API). Four indicators are displayed:

| Indicator | Formula | National Average | Interpretation |
| --- | --- | --- | --- |
| % People of Colour (%PoC) | (Total pop. - Non-Hispanic white) / Total pop. x 100 | 40.0% (ACS 2022) | Communities with %PoC above the national average are considered more demographically vulnerable under EJScreen. |
| % Low-Income (%LI) | Population below federal poverty line / Poverty universe x 100 | 29.0% (ACS 2022) | Reflects economic vulnerability. Low-income communities often bear disproportionate environmental burdens. |
| Demographic Index (DI) | (%PoC + %LI) / 2 | 34.5% | Composite vulnerability indicator used as the EJScreen Demographic Index. |
| Total Population | ACS B01003_001E | - | Total resident population of the zip code tabulation area; used as a multiplier in the EJ Index formula. |

## 9.3  EJ Index Calculation

An EJ Index is computed for each conversion technology using the minimum GHG emissions achieved by that technology across all Pareto solutions. The index formula is:

EJ Index = GHG (t CO2-eq yr-1)  x  (Local DI - National Average DI)  x  Total Population

A higher EJ Index indicates greater cumulative environmental burden placed on a more vulnerable community. Pathways are ranked from lowest to highest EJ Index and classified into three concern levels (Low, Moderate, High) based on their relative position within the range of scores across all evaluated pathways. A negative EJ Index arises when the local DI is below the national average, indicating the facility would be sited in a relatively less vulnerable community.

## 9.4  Recommendation

A plain-language recommendation section identifies the technology pathway with the lowest EJ Index and quantifies the reduction in community burden relative to the worst-performing pathway, expressed in absolute EJ Index units. This output is intended to support environmental justice reporting in permit applications, environmental impact assessments, and regulatory submissions.

# 10.  Mathematical Model Summary

ECO-FAST solves a Mixed-Integer Nonlinear Program (MINLP) comprising binary technology selection variables, continuous mass flow and economic variables, and a set of linear and nonlinear constraints. The complete mathematical formulation with all numbered equations is presented in the Methods section of the associated publication. Key model statistics are summarised below.

| Attribute | Value / Description |
| --- | --- |
| Model type | MINLP (Mixed-Integer Nonlinear Program) |
| Number of material streams | 61 |
| Number of stream components | 8 (WATER, CBH, PRT, FAT, OTH, ASH, FC, ENZYME) |
| Number of processing units | 15 |
| Binary decision variables | ~20 (technology selection and routing switches) |
| Continuous variables | ~500+ (stream flows, capacities, costs, emissions, sizing variables) |
| Equality constraints | ~400 (mass balances, sizing, cost, GHG component equations) |
| Nonlinear terms | Capital cost scaling law (Qc/Q0)^0.67 per unit; steam turbine enthalpy calculation |
| Objective functions | 2: NAC (M$ yr-1) and GHG (t CO2-eq yr-1) |
| Multi-objective method | Epsilon-constraint Pareto frontier (N user-defined points) |
| Solver | SBB (Simple Branch and Bound) via GAMSPy v1.23.1 |
| NLP subproblem solver | CONOPT (local NLP; called at each B&B node) |
| Optimality | Local optimum (SBB is a local MINLP solver; global optimality not guaranteed for nonconvex problems) |
| Relative optimality gap | 5% (default; user-adjustable) |
| Time limit per solve | 300 seconds (default; user-adjustable) |

# 11.  Data Flow Between Tabs

All user inputs are held in Streamlit session state throughout a session. When the user clicks 'Run Optimisation' in Tab 5, a unified input dictionary is assembled from all session state values and sent to the FastAPI backend as a single JSON request. The data flow is strictly sequential across tabs.

| Source Tab | Data Produced | Used In |
| --- | --- | --- |
| Tab 2 - Feed Inputs | Q_f (kg/hr), T_ann (hr/yr), wet-basis composition (7 fractions), zip code | Backend model - all process units use feed flow and composition as boundary conditions. Zip code used in Tab 6 for EJ assessment. |
| Tab 3 - Technology Specifications | Unit-specific engineering parameters for all 15 technologies (yields, efficiencies, HRTs, emission factors) | Backend model - overrides default parameter values in parameters.py for the selected technology parameters. |
| Tab 4 - Cost Specifications | C_elec, C_tip, C_lbr, discount rate r, plant lifetime n, product prices, objective mode, N Pareto points, time limit, gap | Backend model - costing.py (cost equations), solver.py (objective and epsilon grid configuration). |
| Tab 5 - Results | Pareto dataframe: NAC, GHG, all cost and GHG components, pathway configuration per Pareto point | Displayed in Tab 5. GHG values passed to Tab 6 for EJ Index calculation. Full dataframe available as CSV download. |
| Tab 6 - EJ Assessment | EJ Index per pathway, community demographics, facility map | Final output only. Not fed back to any upstream tab. |

# 12.  Software Stack and Deployment

## 12.1  Software Dependencies

| Package | Version | Role |
| --- | --- | --- |
| Python | 3.12 | Runtime environment for all model and application code |
| GAMSPy | 1.23.1 | Python-native algebraic modelling interface to the GAMS solver system |
| GAMS / SBB | System license | Branch-and-bound MINLP solver; SBB is the local MINLP solver used for all optimisation runs |
| Streamlit | Latest stable | Frontend web application framework; renders all six tabs and interactive widgets |
| FastAPI | Latest stable | REST API backend framework; receives optimisation requests and returns results as JSON |
| Uvicorn | Latest stable | ASGI web server that hosts the FastAPI application |
| Pandas | Latest stable | Tabular data handling, Pareto result formatting, and CSV export |
| Matplotlib | Latest stable | Composition donut chart and Pareto frontier scatter plot in the Results tab |
| Folium | Latest stable | Interactive facility location map with buffer zones in the EJ tab |
| streamlit-folium | Latest stable | Streamlit component for rendering Folium maps within the Streamlit interface |
| Requests | Latest stable | HTTP requests to the Zippopotam.us geocoding API and US Census Bureau ACS data API |
| python-docx | Latest stable | Used to generate this report document programmatically |

## 12.2  Starting the Application

Both processes must be started in separate terminal windows. The backend API must be started first, followed by the Streamlit frontend.

Terminal 1 - Backend API:  py -3.12 -m uvicorn api:app --host 127.0.0.1 --port 8000

Terminal 2 - Frontend:  py -3.12 -m streamlit run app.py

Once both processes are running, the application is accessible in a web browser at http://localhost:8501. The API health endpoint can be verified at http://127.0.0.1:8000/health, which returns {"status": "ok"} when the backend is operating correctly.

## 12.3  Sharing with External Stakeholders (Temporary Access)

For sharing the tool with external collaborators or regulatory agencies (e.g. EPA) during a meeting or review period, the application can be temporarily exposed to the internet using the ngrok tunnelling service:

Step 1:  Start both backend and frontend processes as described above.

Step 2:  Run: ngrok http 8501

Step 3:  Share the public URL provided by ngrok (e.g. https://abc123.ngrok-free.app) with stakeholders. All computation remains on the host machine; the GAMS license remains valid on the local installation.

This approach requires no cloud infrastructure and no license transfer. The ngrok free tier displays a one-time warning page before the application loads; a paid ngrok account removes this warning.

## 12.4  Permanent Cloud Deployment

For permanent hosting accessible to all stakeholders at any time, the application can be deployed on a cloud virtual machine (e.g. Google Cloud Compute Engine, AWS EC2, or Azure VM). Both backend and frontend processes run on the same VM, and a public IP address or custom domain provides permanent access.

The key requirement for cloud deployment is a valid GAMS license installed on the cloud VM. Standard GAMS node-locked licenses may be transferred to a cloud VM (up to three transfers per license term under current GAMS EULA terms). Cloud deployment costs are approximately $25-50 per month for a general-purpose VM instance of appropriate size (e.g. 2 vCPUs, 8 GB RAM).